# PXP- Certification

# 4.3 Discover Services and Characteristics

4.3.4 TP/PXS/PM/BV-04-C [Discover Link Loss Service using GATT for LE] ________________________________________________________

Verify that the Link Loss Service can be discovered by the Proximity Monitor IUT.

  Reference

Proximity Profile Specification v1.0 [ 4.1 ]

  Initial Condition

Establish an ATT Bearer connection between the Tester and IUT; see 4.2.

The lower tester includes one instantiation of the Link Loss Service [8].

  Test Procedure

The upper tester issues a command to the IUT to discover primary services. There are two alternatives:

1.  Execute the procedure defined in GATT.TS [6] Discover All Primary Services,

TP/GAD/CL/BV-01-C.

2.  Execute the procedure defined in GATT.TS [6] Discover Primary Services by

Service UUID, TP/GAD/CL/BV-02-C, with the service UUID set to «Link Loss»

  Expected Outcome

Pass verdict

The IUT successfully discovers one instance of the Link Loss Service.

Fail verdict

Otherwise

Inconclusive verdict

N/A

Notes

N/A

Evidence (Screen Shot)

Final verdict

Pass

- IUT discovers Link Loss Service by sending Read Group Type Request.

4.3.5 TP/PXS/PM/BV-05-C [Discover Immediate Alert Service using GATT for LE]

_________________________________________________________

Verify that the Immediate Alert Service can be detected by the Proximity Monitor IUT.

  Reference

Proximity Profile Specification v1.0 [ 4.1 ]

  Initial Condition

Establish an ATT Bearer connection between the Tester and IUT; see 4.2.

The lower tester includes one instantiation of the Immediate Alert Service [7].

  Test Procedure

The upper tester issues a command to the IUT to discover primary services.  There are two alternatives:

1.  Execute the procedure defined in GATT.TS [6] Discover All Primary Services, TP/GAD/CL/BV-01-C.

2.  Execute the procedure defined in GATT.TS [6] Discover Primary Services by Service UUID, TP/GAD/CL/BV-02-C, with the service UUID set to «Immediate

Alert».

  Expected Outcome

Pass verdict

The IUT successfully discovers one instance of the Immediate Alert Service.

Fail verdict

Otherwise

Inconclusive verdict

N/A

Notes

N/A

Evidence (Screen Shot)

Final verdict

Pass

- IUT discovers Immediate Alert Service by sending Read Group Type Request.

4.3.6 TP/PXS/PM/BV-06-C [Discover Tx Power Service using GATT for LE]

__________________________________________________________

Verify that the Tx Power Service can be detected by the Proximity Monitor IUT.

  Reference

Proximity Profile Specification v1.0 [ 4.1 ]

  Initial Condition

Establish an ATT Bearer connection between the Tester and IUT; see 4.2.

The lower tester includes one instantiation of the Tx Power Service [9].

  Test Procedure

The upper tester issues a command to the IUT to discover primary services.  There are two alternatives:

1.  Execute the procedure defined in GATT.TS [6] Discover All Primary Services,

TP/GAD/CL/BV-01-C.

2.  Execute the procedure defined in GATT.TS [6] Discover Primary Services by

Service UUID, TP/GAD/CL/BV-02-C, with the service UUID set to «Tx Power».

  Expected Outcome

Pass Verdict

The IUT successfully discovers one instance of the Tx Power Service.

Fail Verdict

Otherwise

Inconclusive verdict

N/A

Notes

N/A

Evidence (Screen Shot)

Final verdict

Pass

- IUT discovers Tx Power Service by sending Read Group Type Request.

4.3.7 TP/PXS/PM/BV-07-C [Discover Alert Level Characteristic for Link Loss]

Verify that the Alert Level Characteristic for the Link Loss service can be detected by the Proximity Monitor IUT.

  Reference

[3] 4.2

  Initial Condition

Establish an ATT Bearer connection between the Tester and IUT; see 4.2.

The lower tester includes one instantiation of the Link Loss Service [8], which includes the Alert Level Characteristic.

The IUT has executed the procedure defined in TP/PXS/PM/BV-01-C or

TP/PXS/PM/BV-04-C, and has saved the handle range for an instantiation of the Link Loss Service.

  Test Procedure

The upper tester issues a command to the IUT to discover the Alert Level Characteristic

The IUT executes one of the following alternatives:

Execute the procedure defined in GATT.TS [6] Discover Characteristic by UUID, TP/GAD/CL/BV-05-C, with the Service UUID set to «Alert Level Characteristic». Execute the procedure defined in GATT.TS [6] Discover All Characteristics of a Service, TP/GAD/CL/BV-04-C.

  Expected Outcome

Pass verdict

The IUT successfully discovers the Alert Level Characteristic of the Link Loss Service.

Fail verdict

Otherwise

Inconclusive verdict

N/A

Notes

N/A

Evidence (Screen Shot)

Final verdict

Pass

- IUT discovers Alert Level Characteristics of Link Loss Service (Handle Pair 53-54)

4.3.8 TP/PXS/PM/BV-08-C [Discover Alert Level Characteristic for Immediate Alert]

Verify that the Alert Level Characteristic for the Immediate Alert service can be detected by the Proximity Monitor IUT.

  Reference

[3] 4.2

  Initial Condition

Establish an ATT Bearer connection between the Tester and IUT; see 4.2.

The lower tester includes one instantiation of the Immediate Alert Service [7], which includes the Alert Level Characteristic.

The IUT has executed the procedure defined in TP/PXS/PM/BV-02-C or

TP/PXS/PM/BV-05-C, and has saved the handle range for an instantiation of the Immediate Alert Service.

  Test Procedure

The upper tester issues a command to the IUT to discover the Alert Level Characteristic for Immediate Alert Service.

The IUT executes one of the following alternatives:

Execute the procedure defined in GATT.TS [6] Discover Characteristic by UUID, TP/GAD/CL/BV-05-C, with the Service UUID set to «Alert Level Characteristic».

Execute the procedure defined in GATT.TS [6] Discover All Characteristics of a Service, TP/GAD/CL/BV-04-C.

  Expected Outcome

Pass Verdict

The IUT successfully discovers the Alert Level Characteristic of the Immediate Alert Service.

Fail Verdict

Otherwise

Inconclusive verdict

N/A

Notes

N/A

Evidence (Screen Shot)

Final verdict

Pass

- IUT discovers Alert Level Characteristics of Immediate Alert Service (Handle Pair 56-57)

4.3.9 TP/PXS/PM/BV-09-C [Discover Tx Power Level Characteristic]

__________________________________________________________

Verify that the Tx Power Level Characteristic can be detected by the Proximity Monitor IUT.

  Reference

Proximity Profile Specification v1.0 [ 4.1 ]

  Initial Condition

Establish an ATT Bearer connection between the Tester and IUT; see 4.2.

The lower tester includes one instantiation of the Tx Power Service [9], which includes the Tx Power Level Characteristic.

Test Procedure

The upper tester issues a command to the IUT to discover the Tx Power Level Characteristic.

The IUT executes one of the following alternatives:

Execute the procedure defined in GATT.TS [6] Discover Characteristic by UUID, TP/GAD/CL/BV-05-C, with the Service UUID set to

«Tx Power Level Characteristic».

Execute the procedure defined in GATT.TS [6] Discover All Characteristics of a

Service, TP/GAD/CL/BV-04-C.

  Expected Outcome

Pass verdict

The IUT successfully discovers the Tx Power Level Characteristic of the Tx Power Service.

Fail verdict

Otherwise

Inconclusive verdict

N/A

Notes

N/A

Evidence (Screen Shot)

Final verdict

Pass

- IUT discovers Tx Power Level Characteristics of Tx Power Level Service.

# 4.4 Proximity Profile Features

4.4.1 TP/PXF/PM/BV-01-C [Configuration of Alert Level] __________________________________________________________

Verify that the Proximity Monitor can write the Alert Level characteristic value of the Link Loss Service to a Proximity Reporter.

  Reference

[3] Section 4.3

  Initial Condition

Establish an ATT Bearer connection between the Tester and IUT; see 4.2.

  Test Procedure

The upper tester sends a command to the IUT to write a valid Alert Level value to the Alert Level characteristic in the lower tester.

  Expected Outcome

Pass verdict

The IUT sends a correctly formatted ATT_Write_Request to the lower tester, containing the handle of the alert level characteristic of the Link Loss Service, and with the Alert Level value specified by the upper tester.

Fail verdict

Otherwise

Inconclusive verdict

N/A

Notes

N/A

Evidence (Screen Shot)

Final verdict

Pass

-IUT configures Alert Level Characteristics of Link Loss Service.

4.4.2 TP/PXF/PM/BV-02-C [Alert on Link Loss] __________________________________________________________

Verify that when the monitored link is disconnected, the Proximity Monitor alerts.

  Reference

[3] Section 4.4

  Initial Condition

Establish an ATT Bearer connection between the Tester and IUT; see 4.2.

  Test Procedure

The link is disconnected by interrupting the radio communication between the two devices, without sending a disconnection command.

  Expected Outcome

Pass verdict

The IUT starts alerting when the physical link is disconnected. Note that the level of alerting is implementation specific and that some implementations may use No Alert as a valid alert level.

Fail verdict

Otherwise

Inconclusive verdict

N/A

Notes

N/A

Evidence (Screen Shot)

Final verdict

Pass

- After disconnection due to connection timeout IUT starts alerting.

4.4.3 TP/PXF/PM/BV-03-C [Read Tx Power] __________________________________________________________

Verify that the Proximity Monitor can read the Tx Power Level characteristic after establishment of connection.

  Reference

[3] Section 4.5

  Initial Condition

Establish an ATT Bearer connection between the Tester and IUT; see 4.2.

The upper tester knows the handle of a Tx Power Level characteristic contained in the lower tester.

  Test Procedure

1.  Send a command from upper tester to request IUT to read a Tx Power Level Characteristic from the lower tester using either the Read Characteristic Value or Read Using Characteristic GATT procedure.

2.  After receipt of the expected result by the lower tester, send an

ATT_Read_Response from the lower tester to the IUT.

  Expected Outcome

Pass verdict

The IUT sends a correctly formatted and valid Tx Power Level to the upper tester.

Fail verdict

Otherwise

Inconclusive verdict

N/A

Notes

N/A

Evidence (Screen Shot)

Final verdict

Pass

-IUT reads Tx Power level by sending Read Request with Handle value of Tx Power Level.

4.4.5 TP/PXF/PM/BV-05-C [Alert on Exceeding Path Loss] __________________________________________________________

Upon exceeding a path loss determined by the Proximity Monitor, verify that the Proximity Monitor writes the Alert Level characteristic of the Immediate Alert Service.

  Reference

[3] Section 4.6

  Initial Condition

Establish an ATT Bearer connection between the Tester and IUT; see 4.2.

The lower level tester has set its Alert Level characteristic to No Alert.

  Test Procedure

Send a command from the upper tester to request the IUT to trigger an immediate alert.

  Expected Outcome

Pass verdict

The IUT uses the Characteristic Write command to write the Alert Level

characteristic of the Immediate Alert Service in the lower tester with the value of Mild Alert or High Alert.

Fail verdict

Otherwise

Inconclusive verdict

N/A

Notes

N/A

Evidence (Screen Shot)

Final verdict

Pass

-IUT sets alert level to High.

4.4.6 TP/PXF/PM/BV-06-C [Cancel Alert on Reducing Path Loss] __________________________________________________________

After an alert due to exceeding a path loss determined by the Proximity Monitor, as the path loss falls below a threshold determined by the Proximity Monitor, verify that the Proximity Monitor writes the Alert Level characteristic of the Immediate Alert Service

with No Alert.

  Reference

[3] Section 4.6

  Initial Condition

Establish an ATT Bearer connection between the Tester and IUT, see 4.2.

An Alert level value of Mild Alert or High Alert has previously been written to the Alert Level characteristic of the Immediate Alert Service in the lower tester.

  Test Procedure

Send a command from the upper tester to request the IUT to stop the immediate alert.

  Expected Outcome

Pass verdict

The IUT uses the Characteristic Write command to write the Alert Level

characteristic of the Immediate Alert Service in the lower tester with the value No Alert.

Fail verdict

Otherwise

Inconclusive verdict

N/A

Notes

N/A

Evidence (Screen Shot)

Final verdict

Pass

-IUT sets alert level to NONE

4.4.7 TP/PXF/PM/BV-07-C [Verify Bond Status on Reconnection] __________________________________________________________

Verify that the Proximity Monitor starts encryption with a previously bonded Proximity Reporter on reconnection.

  Reference

[3] Section 5.2.3

  Initial Condition

The IUT and the tester are bonded.

No connection is established between the IUT and tester.

  Test Procedure

The lower tester begins advertising using GAP undirected connectable mode.

The IUT establishes a connection to the lower tester.

The IUT starts encryption when the connection is established.

  Test Condition

None

  Expected Outcome

Pass verdict

The IUT starts encryption when the connection is established.

Fail verdict

Any of the pass verdicts fail to occur

Inconclusive verdict

N/A

Notes

N/A

Evidence (Screen Shot)

Final verdict

Pass

-IUT enables encryption immediately after LE connection.